European Commission

EU Reference Laboratory for GM Food

and Feed

Template protocol for

GMO specific real-time PCR system

______________________________________

Protocol for event-specific quantitation

of (specify name of GM event)

Confidential - Not to be circulated.

Contents

1.	General information on the method	3

Purpose and Scope	3

Principle of the method	3

2.	Details on samples and standard curve	3

3.	Details on composition and preparation of reaction mixes	4

4.	Cycling parameters	6

5.	Primers and probes	6

6.	Equipment and materials	6

7.	Reagents, buffers and solutions	7

# General information on the method

### Purpose and Scope

This protocol describes an event-specific real-time quantitative TaqMan® (or specify chemistry used) PCR method for determination of the relative content of event (specify name of the GM event) DNA to total (specify species, e.g. maize, rapeseed etc.) DNA in a sample.

The PCR assay has been optimized for use in an ABI Prism® 7900 sequence detection system (or specify other equipment).

### Principle of the method

For specific detection of event (specify name of the GM event) genomic DNA, a (give amplicon length)-bp fragment of the region that spans the 3’ insert-to-plant junction in  (specify species) event (specify name of the GM event) is amplified using two specific primers. PCR products are measured during each cycle (real-time) by means of a target-specific oligonucleotide probe labeled with two fluorescent dyes: (specify fluorescent dye, e.g. FAM, VIC etc.) as a reporter dye at its 5 end and TAMRA (or specify other quencher dye) as a quencher dye at its 3 end.  The 5-nuclease activity of the Taq DNA polymerase is exploited, which results in the specific cleavage of the probe, leading to increased fluorescence, which is then monitored.

For relative quantification of event (specify name of the GM event) DNA, a (specify species)-specific reference system amplifies a  (give amplicon length)-bp fragment of (give name of endogenous gene), a (specify species) endogenous gene, using a pair of (give name of endogenous gene) gene-specific primers and an (give name of endogenous gene) gene-specific probe labeled with (specify fluorescent dye) and TAMRA (or specify other quencher dye) as described above.

# Details on samples and standard curve

DNA Concentration of samples

For the unknown samples, the use of (specify the amount of total genomic DNA) ng of template DNA per reaction well is recommended. The corresponding concentration and genome copy number are as follows:

| Final quantity per well | XXX |
| --- | --- |
| Concentration (ng/µl) | XX |

DNA Concentration of standard samples

The standard curves comprise a number of (give number of calibration samples) points (from S1 to Sx)

The first standard curve point S1 is derived from a sample containing the following GM % of the event (specify name of the GM event): (please give the GM% of the calibration sample)

Standard curve points S2 – SX are obtained by serial dilution of the X% GM standard S1. The dilution scheme and the corresponding total genomic DNA content in PCR is described below:

| Sample code | S1 | S2 | S3 | S4 | S5 |
| --- | --- | --- | --- | --- | --- |
| Total DNA content in PCR | XXXX | XXXX | XXXX | XXXX | XXXX |

NOTE: Add additional standard points if needed

The concentration range tested and recommended for the standard curve is: (give range expressed as total DNA content and/or GM %).

# Details on composition and preparation of reaction mixes

- If necessary, thaw all reagents. It is recommended to store all reagents on ice once thawed.  Be sure to thoroughly mix each reagent before use.  Two reaction mixes (one for event [specify name of the GM event] PCR and one for [give name of endogenous gene] PCR) must be prepared consisting of all components of the PCR, except DNA template, in sufficient quantities for all reactions (including those for standard DNA solutions) to be performed (i.e., prepare each reaction mix for at least two extra reaction).  This allows for consistency and minimizes cross-contamination

It is recommended that all reagents be added in the order listed below for both reaction mixes.

NOTE: Two examples of reaction mix compositions are given below. Please modify the composition of mixes as appropriate.

Preparation of the master-mix for the reference gene system

| Component | Final concentration in PCR | µl/reaction |
| --- | --- | --- |
| TaqMan Universal Master Mix 2X Primer F (XX µM) Primer R (XX µM) Probe (XX µM) Nuclease free water Template DNA (maximum XX ng) | 1x XXX nM XXX nM 200 nM # # | XX XXX XXX XXX XXX XX |
| Total volume: |  | XX µl |

Preparation of the master-mix for the (specify name of GM event)

| Component | Final concentration in PCR | µl/reaction |
| --- | --- | --- |
| TaqMan Buffer A 10x MgCl2 (e.g. 25 mM) dNTPa XX mM each dUTP XX mM primer F (XX µM) primer R(XX µM) probe (XX µM) AmpErase UNG (1 U/µl) Ampli Taq Gold (5 U/µl) Nuclease free water Template DNA (≤ XX ng DNA for standards, maximum XXX ng for unknowns) | 1x X mM XX mM each XX mM XX nM XX nM XX nM XX U XX U # # | XX X 0XX each XX XXX XXX XXX XXX XXX XX X |
| Total volume: |  | XX µl |

a dNTP = dATP, dCTP and dGTP.

# Cycling parameters

Run the PCR with cycling conditions listed below for both event (specify name of the GM event) and (specify name of the endogenous gene) assays in the ABI Prism 7900 system (or specify other equipment).

Note: please modify as appropriate the cycling parameters given below as an example

| Step | Stage |  | T°C | Time (sec) | Data collection | Cycles |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | UNG |  | 50 °C | 120” | no | 1x |
| 2 | Initial denaturation |  | 95oC | 600” | no | 1x |
| 3 | Amplification | Denaturation | 95oC | 15” | no | 50x |
| 4 |  | Annealing & Extension | 60oC | 60” | yes |  |

# Primers and probes

Two types of quantitation are performed in this method: one for the endogenous gene (specify name of endogenous gene) and one for the (specify name of GM event) specific sequence region.

The following sets of primers and probes are used:

|  | Sequence (5’ to 3’) |
| --- | --- |
| (GM Event) primer F | XXXXXXXXXXXXXXXXX |
| (GM Event) primer R | XXXXXXXXXXXXXXXXX |
| (GM Event) probe | 6-FAM-XX-TAMRA (or specify other labeling and quenching dyes) |
| (Endogenous) primer F | XXXXXXXXXXXXXXXXX |
| (Endogenous) primer R | XXXXXXXXXXXXXXXXX |
| (Endogenous) probe | 6-FAM-XX-TAMRA (or specify other labeling and quenching dyes) |

Note: primers and probes sequences will not be included in the validation protocol distributed to the participants.

# Equipment and materials

The following equipment may be used in this procedure:

(please provide a detailed list of equipment, analysis software and materials needed; an example is given below):

| Equipment | Number/Specification |
| --- | --- |
| ABI Prism 7700 sequence detection system | Applied Biosystems Part No. 7700-01-200/208 |
| Software: Sequence Detection System version 1.7 | Applied Biosystems Part No. 4311876 |
| ABI Prism 7900HT sequence detection system | Applied Biosystems Part No. 4329002 or 4329004 |
| MicroAmp optical 96-well reaction plates | Applied Biosystems Part No. N801-0560 |
| MicroAmp optical caps (8 caps/strip) | Applied Biosystems Part No. N801-0935 |
| ABI PRISM optical adhesive covers | Applied Biosystems Part No. 4311971 |
| ABI PRISM optical adhesive cover starter kit | Applied Biosystems Part No. 4313663 |
| ABI PRISM optical cover compression pads | Applied Biosystems Part No. 4312639 |

# Reagents, buffers and solutions

The following reagents, buffers and solutions are used in this procedure:

(please provide a detailed list of reagents and buffers, including procedure for buffer and solution preparation and storage; an example is given below):

| Reagent | Number/Specification |
| --- | --- |
| - 0.5 M EDTA | - Sigma Cat. No. E-7889 |
| Nuclease-free water | Sigma Cat. No. W-4502 |
| PCR primers (10 M) and fluorescent oligonucleotide probes (5 M) | Synthesized by (specify company) |
| TaqMan universal PCR master mix (2X) | Applied Biosystems Part No. 4304437 |
| - 1 M Tris-HCl, pH 8.0 | - Sigma Cat. No. T-3038 |

Example for the preparation of TE buffer:

| - TE Buffer, - pH 7.0 | - 10 mM Tris, 1 mM EDTA, pH 7.0 - For 250 ml: - Mix 100 ml of nuclease-free water, 2.5 ml of 1 M Tris, pH 8.0 and 0.5 ml of 0.5 M EDTA. - Adjust pH to 7.0 with HCl. - Adjust final volume to 250 ml with nuclease-free water. - Filter sterilise. Store at room temperature for up to 5 years. |
| --- | --- |